Lab 1: Introduction to SCADA/ICS

Objectives

- Understand the basics of SCADA (Supervisory Control and Data Acquisition) and ICS (Industrial Control Systems).

- Explore the key industrial sectors utilizing SCADA/ICS technologies.

- Recognize the importance of SCADA/ICS security.

Prerequisites

- Basic knowledge of computer networks and industrial operations.

- Familiarity with fundamental cybersecurity concepts.

Introduction

In this lab, we will explore the foundational concepts of SCADA and ICS, focusing on their roles in industrial automation and control systems. We will discuss the need for securing these systems and analyze their use across various sectors.

Tasks

Task 1: Define SCADA and ICS

1.1 Understand SCADA

- SCADA stands for Supervisory Control and Data Acquisition. It is a system used for controlling industrial processes and collecting real-time data.

- Key Features of SCADA:

  - Real-time data acquisition

  - Remote control capabilities

  - Data storage and management

Example: A SCADA system can manage a power grid where it monitors electricity flows and adjusts outputs from power stations based on demand.

1.2 Understand ICS

- ICS encompasses systems used for controlling and monitoring industrial processes. These include DCS (Distributed Control Systems), PLC (Programmable Logic Controllers), and SCADA systems.

- Types of ICS:

  - Distributed Control Systems (DCS)

  - Programmable Logic Controllers (PLC)

  - SCADA systems

1.3 Key Differences

- Scope: SCADA is focused on data acquisition and control at a supervisory level, while ICS includes a variety of control systems.

Task 2: Identify Key Industrial Sectors Using SCADA/ICS

2.1 Sectors

- Energy Sector: Utilizes SCADA for managing power plants and distribution networks.

- Water and Wastewater: Uses SCADA for water treatment and distribution.

- Manufacturing: Employs ICS for automation of production processes.

Case Study:

A water treatment facility uses SCADA to ensure proper treatment levels, monitor chemical balances, and automate pump operations.

Task 3: Discuss Why SCADA/ICS Security is Critical

3.1 Importance of Security

- Vulnerability: SCADA/ICS systems are often targeted by cyber-attacks due to their critical roles.

- Impacts: Breaches can lead to severe disruptions, environmental damage, and safety hazards.

3.2 Security Strategies

- Network Segmentation: Isolating critical SCADA/ICS networks from other business networks.

- Regular Updates: Ensuring all systems and software are up to date with the latest security patches.

3.3 Example Incident

Example: The Stuxnet worm that targeted Iranâs nuclear facilities, demonstrating how vulnerabilities in ICS systems can be exploited.

Lab Name: ICS/SCADA Architecture Basics

Objectives

- Understand the foundational layers of ICS/SCADA systems: Field, Control, Supervisory, and Enterprise.

- Identify key components such as RTUs, PLCs, and HMIs.

- Develop skills to sketch a basic ICS/SCADA architecture diagram.

Prerequisites

- Basic understanding of industrial control systems and networking.

- Familiarity with basic IT concepts and terminologies.

- Access to an open-source diagramming tool such as Dia or Draw.io.

Lab Tasks

Task 1: Review Typical Layers in ICS/SCADA Systems

1.1 Overview of ICS/SCADA Layers

- Field Layer: Interface directly with physical processes through sensors and actuators. They collect raw data from the environment and transmit it to the control layer.

- Control Layer: Utilizes devices like Programmable Logic Controllers (PLCs) to process the data and make real-time decisions. These are crucial for maintaining system stability.

- Supervisory Layer: Encompasses Supervisory Control and Data Acquisition (SCADA) systems, providing data visualization and operational oversight.

- Enterprise Layer: Bridges the operational technology with information technology, using data analytics and optimizing production processes for business benefits.

1.2 Identify Components

- Remote Terminal Units (RTUs): Collect sensor data from field devices.

- Programmable Logic Controllers (PLCs): Execute control actions based on predefined logic.

- Human-Machine Interfaces (HMIs): Allow human operators to interact with the SCADA system through graphical interfaces.

Case Study: Review a sample architecture of a water treatment plant that uses these layers effectively.

Task 2: Identifying RTUs, PLCs, and HMIs

2.1 Analysis of Components

- RTUs:

  - Primarily used in environments where standard PLCs cannot be deployed due to environmental challenges.

  - Example: RTUs in an oil pipeline monitoring application.

- PLCs:

  - Essential for automation tasks requiring precise control logic.

  - Example: A PLC used in robotic assembly lines in automobile manufacturing.

- HMIs:

  - Facilitate decision-making by providing real-time data.

  - Example: HMI screens at a power generation facility showing turbine performance metrics.

2.2 Hands-On Identification

- Use an open-source tool like OpenPLC Editor to simulate how a PLC works.

# Sample OpenPLC Editor Command

# Define an LED toggle logic

PROGRAM PLC_PRG

VAR

LED : BOOL;

END_VAR

LED := NOT LED;

END_PROGRAM

Task 3: Sketch a Simple ICS/SCADA Architecture Diagram

3.1 Tools and Techniques

- Choose an open-source tool like Draw.io for diagramming.

- Focus on accuracy and clarity by using standardized symbols for each component.

3.2 Sketching Activity

- Create a flow that includes the Field layer (sensors/actuators), Control layer (PLCs), Supervisory layer (HMIs), and the Enterprise layer (business network systems).

Case Study Implementation:

- Sketch the water treatment plant architecture as a practical application of the layers discussed.

3.3 Save and Review the Diagram

- Ensure the diagram highlights data flow and control logic paths clearly.

Lab Name: 3. ICS/SCADA Terminology

Objectives

- Understand the key terminology used in ICS/SCADA systems.

- Differentiate between ICS and traditional IT systems.

- Explore real-world examples of ICS usage.

Prerequisites

- Basic understanding of computer systems and networks.

- Familiarity with the concept of industrial control systems.

Lab Tasks

Task 1: Compile a Glossary of Terms

Subtask 1.1: Define Key Terms

- PLC (Programmable Logic Controller)

  - Description: A digital computer used for automation of electromechanical processes, such as control of machinery on factory assembly lines.

  - Example: Used in batch processing for chemical production.

- RTU (Remote Terminal Unit)

  - Description: A microprocessor-controlled electronic device that interfaces objects in the physical world to a distributed control system or SCADA by transmitting telemetry data to a master system.

  - Example: Used in remote monitoring of oil pipelines.

- HMI (Human-Machine Interface)

  - Description: Software application that presents information to an operator about the state of a process and accepts and implements the operatorâs control instructions.

  - Example: Touch screen interfaces for controlling processes in manufacturing plants.

- DCS (Distributed Control System)

  - Description: A control system where controller elements are not central in location but are distributed throughout the system with each component sub-system controlled by one or more controllers.

  - Example: Used in power plants for precise control and management.

- SCADA (Supervisory Control and Data Acquisition)

  - Description: A control system architecture comprising computers, networked data communications, and graphical user interfaces for high-level process supervisory management.

  - Example: Utilized in water treatment and distribution plants.

Task 2: Explain the Difference between ICS and Traditional IT Systems

- Industrial Control Systems (ICS) are systems composed of control components such as electrical, mechanical, hydraulic, and pneumatic devices that work together to achieve a specific functionality within an industrial process.

- Difference:

  - Real-time Operations: ICS operate in real-time, which is crucial for safety and operational continuity.

  - Safety Requirements: ICS systems prioritize safety over security, unlike IT systems.

  - Lifecycle: ICS have a longer lifecycle than traditional IT systems.

  - Data Integrity vs. Availability: In IT, data integrity and confidentiality are priorities, whereas availability and operational continuity are paramount in ICS.

Task 3: Discuss Real-World Examples of ICS Usage

- Case Study: Smart Grid System

  - Description: Utilizes ICS/SCADA systems to monitor and manage electric power networks, enabling real-time control and optimization of power distribution.

  - Components: Include RTUs and PLCs for controlling substations and distribution lines, along with HMIs for operators.

- Case Study: Water Treatment Plant

  - Description: Employs SCADA systems to automate and monitor water treatment processes. The integration of DCS ensures precise management of chemical levels across multiple stages.

  - Impact: Improves efficiency and reliability while reducing human error.

Lab 4: ICS/SCADA Communication Protocols Overview

Objectives

- Understand the key communication protocols used in Industrial Control Systems (ICS) and Supervisory Control and Data Acquisition (SCADA) systems.

- Distinguish between serial-based and Ethernet-based ICS protocols.

- Identify practical applications and usage of these protocols in industrial settings.

Prerequisites

- Basic understanding of ICS/SCADA systems.

- Familiarity with TCP/IP networking concepts.

- Access to a networked lab environment (simulation or real) with the capability to run virtual machines or containers.

Lab Tasks

Task 1: Introduction to ICS/SCADA Protocols

- List the major protocols used in ICS/SCADA systems:

  - Modbus

  - DNP3 (Distributed Network Protocol 3)

  - OPC (OLE for Process Control)

- Explanation:

  - ICS/SCADA protocols are critical for facilitating communication between various devices in industrial environments. These protocols can be either serial-based or Ethernet-based, affecting their application and efficiency.

Task 2: Distinguishing Serial-based and Ethernet-based Protocols

2.1 Serial-based Protocols

- Modbus RTU: A serial communication protocol primarily used for connecting industrial electronic devices over a RS-485/RS-232 interface.

  - Example: Connects numerous devices, like sensors and PLCs, to a central controller.

- Code Snippet Example: Simulating Modbus RTU communication with Python using pymodbus library.

- from pymodbus.client.sync import ModbusSerialClient as ModbusClient

- # Initialize client

- client = ModbusClient(method='rtu', port='/dev/ttyUSB0', baudrate=9600, timeout=1)

- # Connect to the client

- client.connect()

- # Read from a register

- result = client.read_holding_registers(1, 1, unit=1)

- print(f'Register Value: {result.registers}')

- # Close the connection

- client.close()

2.2 Ethernet-based Protocols

- Modbus TCP/IP: An extension of Modbus RTU for Ethernet, allowing integration into modern networks.

- DNP3: Predominantly used in the utilities sector for SCADA applications and known for its robust communication capabilities over Ethernet.

- OPC UA: A platform-independent service-oriented architecture that allows various industrial hardware and software to communicate through Ethernet.

- Code Snippet Example: Reading data over Modbus TCP using Python:

- from pymodbus.client.sync import ModbusTcpClient

- # Initialize client

- client = ModbusTcpClient('192.168.1.100')

- # Connect to the client

- client.connect()

- # Read from a register

- result = client.read_holding_registers(1, 1)

- print(f'Register Value: {result.registers}')

- # Close the connection

- client.close()

Task 3: Identifying Protocol Applications in a Plant

- Modbus: Commonly used in manufacturing for process monitoring and control.

- DNP3: Often found in the energy sector, especially in electricity and water plant automation.

- OPC UA: Facilitates communication between systems from different vendors, widely used in industries for interoperability.

- Case Study Example: A water treatment plant uses a combination of Modbus and DNP3:

  - Modbus: Monitors the flow rate and valve positions in the treatment process.

  - DNP3: Manages the communication with remote sensors and pumps, ensuring reliable data exchange over long distances.

Lab 5: Setting Up a Virtual ICS/SCADA Lab Environment (Vagrant not running properly)

Objectives

- Understand the basics of virtualization and how it can be applied to create a virtual ICS/SCADA lab environment.

- Install and configure open source virtualization software.

- Deploy a virtual machine to simulate ICS functionalities.

- Configure basic network settings within the virtual environment.

Prerequisites

- Basic understanding of ICS/SCADA systems.

- Familiarity with command-line interface operations.

- A computer with a minimum of 8GB RAM and 50GB free disk space.

- Internet connectivity for downloading necessary software.

Lab Tasks

1. Install Virtualization Software

- Step 1.1: Download and install VirtualBox

  - Navigate to the VirtualBox download page.

  - Select the installer for your operating system and download it.

  - Follow the installation wizard to complete the installation process.

- Step 1.2: Install Vagrant to simplify VM management

  - Download Vagrant from the Vagrant download page.

  - Install Vagrant by running the installer appropriate for your OS.

- # For Linux (Ubuntu)

- sudo apt-get update

- sudo apt-get install -y virtualbox

- sudo apt-get install -y vagrant

2. Set Up a Simple VM for ICS Functionality

- Step 2.1: Initialize a new Vagrant VM

  - Open your terminal or command prompt.

  - Create a new directory for the Vagrant environment:

- mkdir ICS_Lab

- cd ICS_Lab

- vagrant init ubuntu/bionic64

- Step 2.2: Configure the Vagrantfile

  - Open the Vagrantfile in a text editor and modify the following configurations:

- Vagrant.configure("2") do |config|

- config.vm.box = "ubuntu/bionic64"

- config.vm.network "public_network"

- config.vm.provider "virtualbox" do |vb|

- vb.memory = "2048"

- vb.cpus = 2

- end

- end

- Step 2.3: Start the VM

  - Run the following command to launch your virtual machine:

- vagrant up

3. Configure Basic Network Settings

- Step 3.1: Access the VM

  - SSH into the VM using Vagrant:

- vagrant ssh

- Step 3.2: Configure network interfaces

  - Once inside the VM, modify the network settings:

- # View current network interfaces

- ip a

- # Edit the network configuration file

- sudo nano /etc/netplan/01-netcfg.yaml

- # Sample configuration for static IP (replace with your network details)

- network:

- version: 2

- ethernets:

- eth0:

- dhcp4: no

- addresses:

- - 192.168.1.100/24

- gateway4: 192.168.1.1

- nameservers:

- addresses: [8.8.8.8, 8.8.4.4]

- # Apply changes

- sudo netplan apply

Case Study: Simulating Human-Machine Interface (HMI)

- Use open-source SCADA/HMI software like Rapid SCADA to simulate real-world ICS scenarios.

- Rapid SCADA Installation: Follow the official installation guide for a basic setup.

Lab Name: Basic Network Segmentation Concepts

Objectives:

- Understand the concept of network segmentation and its importance in cybersecurity.

- Learn how to design a DMZ (Demilitarized Zone) within a network.

- Develop skills to separate corporate and Industrial Control Systems (ICS) networks.

- Create a simple diagram illustrating segmented ICS and enterprise networks.

Prerequisites

- Basic understanding of networking concepts

- Access to a network simulation tool like GNS3 or Cisco Packet Tracer (open-source alternatives preferred)

- Familiarity with network diagrams

Lab Tasks

Task 1: Review the Concept of a DMZ (Demilitarized Zone)

- Introduction to DMZ:

  - A DMZ is a physical or logical subnetwork that exposes an organization's external-facing services to an untrusted network, commonly the Internet. This serves as an additional layer of security.

- Purpose of a DMZ:

  - Enhance security by isolating external nodes (such as web servers) from internal network resources.

  - Limit the potential impact of a compromised service on internal systems.

- Example Setup:

  - Consider a company web server placed in a DMZ, accessible from the Internet but separate from internal databases and services.

- Case Study:

  - Investigate a real-world example of a DMZ implementation in a small business, noting challenges and benefits discussed.

Task 2: Identify Ways to Separate Corporate and ICS Networks

- Understanding ICS:

  - Industrial Control Systems are systems used to control industrial processes. They often require a distinct network due to unique security needs.

- Network Segmentation Techniques:

  - Firewalls: Use firewalls to control traffic between corporate and ICS networks.

  - VLANs (Virtual LANs): Create VLANs to logically separate networks within the same physical infrastructure.

  - Air-Gapping: Physically or logically disconnect networks to prevent unauthorized access.

- Scenario Analysis:

  - Consider a manufacturing plant using separate VLANs to protect its ICS from corporate network traffic.

Task 3: Create a Simple Diagram Showing Segmented ICS and Enterprise Networks

- Designing the Network:

  - Use a network design tool like Draw.io (open-source alternative) to create a simple diagram.

  - Illustrate a corporate network connected to a public network through a DMZ. Show a separate ICS network segmented via a firewall or VLAN.

- Diagram Components:

  - External Users: Represented on one side accessing a web server in the DMZ.

  - DMZ: Illustrate using separate icons for web server/s and any additional service split from internal networks.

  - Corporate Network: Include typical devices like workstations and file servers, showing distinct separation from ICS.

  - ICS Network: Illustrate isolated from other networks, connected through controlled access points like firewalls.

- Security Considerations:

  - Highlight how firewalls or VLANs enforce security policies.

  - Discuss potential attack vectors and mitigation strategies.

Lab Name: Identifying ICS Components in a Network

Objective:

- Understand and identify various Industrial Control System (ICS) components within a network using open-source tools.

- Develop skills to recognize and categorize ICS hosts such as Programmable Logic Controllers (PLCs) and Human-Machine Interfaces (HMIs).

- Practice documenting network inventory effectively.

Prerequisites:

- Basic understanding of ICS architecture.

- Familiarity with network scanning and the use of tools like nmap.

- Access to a lab environment with ICS components simulated.

Lab Tasks:

Task 1: Network Scanning Using Nmap

Step 1.1: Install Nmap

- Objective: Ensure nmap is installed on your system.

- Command:

- sudo apt-get update

- sudo apt-get install nmap

Step 1.2: Conduct a Basic Network Scan

- Objective: Identify connected devices in the network.

- Command:

- nmap -sn 192.168.1.0/24

- Explanation: This command performs a "ping scan," identifying live hosts without port scanning.

Task 2: Identify ICS Components

Step 2.1: Analyze Nmap Results

- Objective: Review the scan output for ICS components.

- Key Concepts:

  - PLCs: May appear with vendor-specific names (e.g., Siemens, Allen-Bradley).

  - HMIs: Often run on PCs or embedded devices, identifiable by ports and service banners.

Step 2.2: Deep Scan Specific Devices

- Objective: Obtain more detailed information on potential ICS devices.

- Command:

- nmap -A 192.168.1.10

- Explanation: The -A switch enables OS detection, version detection, script scanning, and traceroute.

Task 3: Categorize and Document Findings

Step 3.1: Categorize Discovered Hosts

- Objective: Organize detected devices into categories.

- Approach:

  - Create a spreadsheet or a document to log device details.

  - Columns may include: IP Address, Hostname, Device Type (PLC, HMI, etc.), Operating System.

Example Table Structure:

| IP Address   | Hostname     | Device Type | Operating System |

|--------------|--------------|-------------|------------------|

| 192.168.1.2  | PLC_Siemens  | PLC         | RTOS             |

| 192.168.1.5  | HMI_Station1 | HMI         | Windows Embedded |

Case Study: Practical Scenario

- Scenario: An ICS network in a manufacturing plant is being assessed.

- Approach: Use steps from the lab to scan the network, identify each device, and prepare an inventory. Discuss potential findings such as a misconfigured PLC.

Lab 8: Windows vs. Industrial OS Basics

Objectives

- Understand the differences between Windows and industrial Operating Systems (OS).

- Identify common OSes used in Industrial Control Systems (ICS).

- Compare the security posture of ICS OS versus standard desktop OS.

- Discuss the challenges associated with updating and patching in industrial environments.

Prerequisites

- Basic understanding of operating systems and computer networks.

- Familiarity with terms such as RTOS, ICS, and Linux.

Task 1: Identify OSes Frequently Used in ICS

Step 1: Introduction to ICS Operating Systems

- Windows XP Embedded: A version of Windows designed for embedded systems; often used due to its ease of integration with Windows-based networks.

- RTOS (Real-Time Operating System): Designed to process data in real-time, commonly used in environments where timing is critical.

- Linux-based Systems: Popular for its open-source nature and flexibility.

Step 2: Case Study: Real-world Examples

- Siemensâ use of Windows XP Embedded in its PLCs (Programmable Logic Controllers).

- VxWorks RTOS used in automotive and aerospace industries.

- The role of Linux in SCADA (Supervisory Control and Data Acquisition) systems for utilities control.

Task 2: Compare Security Posture

Step 1: Security Attributes Analysis

- ICS OS: Focused on reliability and deterministic behavior; may have outdated security protocols due to long life cycles and stability concerns.

- Standard Desktop OS: Regularly updated for security; designed for multitasking and broader use.

Key Security Concepts

- Patch Management: Regular updates in desktop OS versus controlled updates in ICS due to potential disruptions.

Step 2: Highlight Key Differences

| Feature | ICS OS | Standard Desktop OS |
| --- | --- | --- |
| Update Cycle | Longer (risk of disruptions) | Frequent and regular |
| Vulnerabilities | Often based on outdated systems | Usually patched rapidly |

Task 3: Discuss Update/Patch Challenges

Step 1: Identifying Challenges

- Compatibility: Ensuring updates do not interfere with existing industrial applications or hardware.

- Downtime: Many industrial systems are required to run continuously, making downtime for updates a significant concern.

Step 2: Strategies to Overcome Challenges

- Use of redundant systems to allow one to be updated while the other maintains operations.

- Comprehensive testing before deployment of patches in ICS environments.

Example Scenario

- An oil refinery utilizing a Linux-based SCADA may test patches on a clone of the production environment to limit risk.

Lab Name: 9. Basic ICS/SCADA System Exploration

Objectives

- Understand the basic components and functionalities of an ICS/SCADA system.

- Explore and interact with a PLC simulator to gather hands-on experience.

- Learn to configure a simulated device and document the configuration process.

Prerequisites

- Basic understanding of ICS/SCADA systems.

- Familiarity with the concept of PLCs (Programmable Logic Controllers).

- A computer with Java installed (required for running certain simulators).

Tools Required

- ModbusPal: An open-source Modbus simulator that enables testing and development of Modbus slave devices.

Lab Tasks

Task 1: Install or Launch a Basic PLC Simulator

- Download ModbusPal:

  - Navigate to the ModbusPal GitHub repository.

  - Download the latest release of ModbusPal.

- Install Java (if not already installed):

  - Ensure Java Runtime Environment (JRE) is installed on your system.

  - You can download it from the Java official site.

- Launch ModbusPal:

  - Extract the downloaded ModbusPal ZIP file.

  - Open a terminal or command prompt and navigate to the extracted directory.

  - Execute the following command to launch ModbusPal:

- java -jar ModbusPal.jar

Task 2: Interact with the Interface (Read/Write Data Points)

- Create a New Device:

  - Click on File > New to create a new device within ModbusPal.

  - Choose the device type and configure its parameters as needed.

- Read Data Points:

  - Use the Modbus Master functionality to read data points from your configured device.

  - Navigate to Modbus Master and input the necessary parameters such as the address range.

- # Example Modbus master read command (hypothetical)

- modbus_read -a 0 -l 10

- Write Data Points:

  - Modify values within the device registers using ModbusPal.

  - Utilize the Modbus Request Editor to write new values to specific addresses.

- # Example command to write data (hypothetical)

- modbus_write -a 10 -v 1234

Task 3: Document the Steps to Configure a Simulated Device

- Configure Device Parameters:

  - Open the Device Editor and specify settings such as device ID, IP address, and port.

  - Save the configuration for future reference.

- Compile Documentation:

  - Document each configuration step performed in the device setup process.

  - Include screenshots or logs as evidence of configuration settings.

Lab Name: 10. ICS/SCADA Data Flow

Objectives:

- To understand and trace the data flow in Industrial Control Systems (ICS) and Supervisory Control and Data Acquisition (SCADA) systems.

- To identify potential bottlenecks or weak points in data flow.

- To create a simple flow diagram to visualize data movement.

Prerequisites:

- Basic understanding of industrial control systems and network topology.

- Familiarity with open-source network monitoring tools and diagramming software.

- Access to a simple ICS/SCADA simulated environment for tracing data flow.

Introduction

In this lab, you will learn how data is transmitted in ICS/SCADA environments from sensors to Programmable Logic Controllers (PLCs) and then to SCADA systems. Understanding this flow is crucial for diagnosing issues and enhancing the efficiency and security of industrial operations.

Lab Tasks

Task 1: Trace Data Flow from Sensor to PLC

- Understand the Components:

  - Sensors: Devices that collect data from the physical environment.

  - PLCs: Computers used in industrial settings to automate and control equipment.

- Set up the Simulation Environment:

  - Use an ICS/SCADA simulation tool like OpenPLC for modeling data flow.

  - Ensure sensors are properly connected to the network and feeding data to the PLC.

- Trace Data Movement:

  - Use a network monitoring tool like Wireshark to capture traffic between the sensor and PLC.

  - Note: Look for industrial protocols such as Modbus, DNP3, or BACnet.

- Analyze the Captured Data:

  - Examine the captured packets. Observe how sensor data (e.g., temperature, pressure) is formatted and transferred to the PLC.

- # Example: Starting a Wireshark capture

- tshark -i eth0 -Y "modbus" -w modbus-traffic.pcap

Task 2: Trace Data Flow from PLC to SCADA System

- Understand SCADA System Components:

  - SCADA systems allow for supervisory control and data acquisition.

- Monitor PLC and SCADA Communication:

  - Continue using Wireshark, now focusing on traffic between the PLC and SCADA.

  - Identify different types of packets exchanged.

- Analyze Data Interaction:

  - Look for responses and control commands carried over the network.

- # Example capture for examining PLC-to-SCADA communication

- tshark -i eth0 -Y "dnp3" -w dnp3-traffic.pcap

Task 3: Identify Potential Data Bottlenecks or Weak Points

- Review the Flow:

  - Analyze collected data for delays, packet loss, or abnormal patterns.

- Identify Weak Points:

  - Look for bottlenecks like limited bandwidth or high latency at various points in the path from sensor to SCADA.

- Document Findings:

  - Create a brief report detailing any observed inefficiencies and potential improvements.

Task 4: Create a Simple Flow Diagram of Data Movement

- Visualize the Flow:

  - Use a diagramming tool like Dia or draw.io to map the path data takes.

  - Include components like sensors, PLCs, and SCADA systems with arrows showing data direction.

- Diagram Features:

  - Indicate key protocols used (e.g., Modbus, DNP3).

  - Highlight any problematic areas identified in Task 3.

Lab 11: Physical Security in ICS/SCADA

Objectives

- Understand the importance of physical security in Industrial Control Systems (ICS) and Supervisory Control and Data Acquisition (SCADA) environments.

- Identify common physical security measures and their roles.

- Assess potential vulnerabilities in a hypothetical ICS site.

- Develop strategies to mitigate physical security risks.

Prerequisites

- Basic understanding of ICS/SCADA operations.

- Familiarity with general security concepts.

Lab Tasks

Task 1: Understanding Common Physical Safeguards

1.1 Identify Physical Safeguards

- Locks: Discuss different types of locks, their usage, and effectiveness.

  - Example: Keycard locks allow access only to authorized personnel.

- Fences: Explore the significance of perimeter fencing in deterring unauthorized access.

- CCTV (Closed Circuit Television): Explain how CCTV systems contribute to monitoring and security.

1.2 Case Study: Effective Use of Physical Safeguards

- Examine a real-world example where physical safeguards effectively protected an ICS installation.

Task 2: Analyzing Risks of Unauthorized Physical Access

2.1 Risks Overview

- Highlight risks associated with unauthorized access to ICS/SCADA systems.

  - Key Concept: Unauthorized physical access can lead to data breaches, equipment damage, and operational disruptions.

2.2 Example Scenario: Security Breach

- Consider a scenario where an intruder gains access to a control room, leading to system shutdown.

Task 3: Inspecting a Hypothetical ICS Site Plan for Vulnerabilities

3.1 Site Plan Analysis

- Examine a hypothetical site plan for an ICS facility.

- Identify potential physical vulnerabilities, such as unguarded entrances and insufficient surveillance.

3.2 Mitigation Strategies

- Propose strategies to address identified vulnerabilities.

  - Example: Install electronic access control systems at all entry points.

Task 4: Implementing Security Measures

4.1 Open Source Security Tools

- Explore open-source tools that can assist in enhancing physical security.

  - Surveillance Control Software: Integration of open-source CCTV management systems like ZoneMinder.

    - Example Command: Installation and basic setup command for ZoneMinder on a Linux system.

sudo apt-get install zoneminder

Lab 12: Basic ICS/SCADA Vulnerability Review

Objectives

- Understand key vulnerabilities affecting Industrial Control Systems (ICS) and Supervisory Control and Data Acquisition (SCADA) systems.

- Learn how to identify and analyze vulnerabilities using public sources.

- Explore mitigation strategies to secure ICS/SCADA environments.

Prerequisites

- Basic understanding of ICS/SCADA systems.

- Familiarity with cybersecurity principles.

- Access to a device with internet connectivity.

Task 1: Review a Known Vulnerability List

Step 1: Access ICS-CERT Advisories

- Navigate to the ICS-CERT website or its successor CISA ICS webpage. These advisories contain a wealth of information on vulnerabilities impacting ICS devices.

Step 2: Understand the Structure of Advisories

- Most advisories will list:

  - CVE (Common Vulnerabilities and Exposures) identifiers.

  - Affected vendors and products.

  - Description of the vulnerability.

  - Potential impacts.

Step 3: Analyze an Example Advisory

- For instance, review the advisory for CVE-2020-25149:

  - Description: A buffer overflow vulnerability.

  - Impact: Could allow a remote attacker to execute arbitrary code.

  - Systems Affected: Example Vendor's Control System v1.2.3.

Task 2: Identify Top 3 Vulnerabilities in ICS Devices

Step 1: Research Common Vulnerabilities

- Explore general ICS vulnerabilities such as unauthorized access, data leakage, and buffer overflows.

Step 2: Document the Top 3 Vulnerabilities

- Unauthorized Access

  - Description: Attackers gaining unauthorized control.

  - Example: Improper access controls allowing entry to supervisory systems.

- Data Leakage

  - Description: Exposure of confidential information.

  - Case Study: Data exfiltration from water treatment SCADA systems.

- Buffer Overflow

  - Description: Execution of arbitrary commands due to improper memory handling.

  - Example: Historical cases affecting energy sector ICSs.

Task 3: Summarize Methods to Mitigate These Vulnerabilities

Step 1: Develop a Mitigation Plan for Unauthorized Access

- Access Controls: Implement robust authentication and authorization mechanisms.

- Network Segmentation: Use firewalls to isolate ICS networks from corporate IT.

Step 2: Create Strategies to Prevent Data Leakage

- Data Encryption: Encrypt sensitive data in transit and at rest.

- Regular Audits: Conduct frequent security audits and vulnerability assessments.

Step 3: Buffer Overflow Prevention

- Code Security: Employ secure coding practices and input validation.

- Software Updates: Regularly apply patches and update software to fix known issues.

Conclusion

- By thoroughly examining and understanding the vulnerabilities outlined, you can bolster the security of ICS/SCADA systems.

- Regular updates, robust security policies, and proactive monitoring are key to maintaining a secure ICS environment.

Additional Resources

- CISA ICS Advisories

- ICS-CERT PDF Reports

Use this lab material to build a strong foundation in identifying and mitigating ICS/SCADA vulnerabilities, ensuring your systems can withstand potential cyber threats.

Lab 13: Introduction to ICS/SCADA Threats

Objectives

- Understand the difference between external and internal threats in ICS/SCADA environments.

- Analyze notable ICS cyber incidents including Stuxnet and BlackEnergy.

- Identify potential motivations for cyber attackers targeting ICS/SCADA systems.

Prerequisites

- Basic understanding of network security and industrial control systems.

- Familiarity with basic cybersecurity concepts.

Lab Tasks

Task 1: Understanding ICS/SCADA Threats

1.1 External vs. Internal Threats

- External Threats: These originate outside the organization. They may include cyberattacks from hackers, competitors, or nation-state actors. The objective often is data theft, disruption, or sabotage.

- Internal Threats: Arise from within the organization and may include disgruntled employees, contractors with internal access, or accidental negligence leading to security breaches.

Key Concept: An internal threat could be as simple as an employee unknowingly clicking a phishing link, compromising network security.

Task 2: Case Studies of ICS Cyber Incidents

2.1 Analysis of Stuxnet

- Overview: Stuxnet is a computer worm first discovered in 2010 and was designed to target Iran's nuclear enrichment facilities.

- Technical Details: It exploited zero-day vulnerabilities in Windows systems to spread itself and specifically targeted Siemens Step7 software.

- Key Technical Terms:

- - Zero-day vulnerabilities: Previously unknown security flaws.

- - PLC (Programmable Logic Controllers): Digital computers used for automation of electromechanical processes.

- Impact: It reportedly caused physical damage to centrifuges, delaying Iran's nuclear program.

2.2 BlackEnergy Incident

- Overview: BlackEnergy is a toolkit used to conduct high-profile cyberattacks on Ukrainian power stations in 2015.

- Method of Attack: Used spear-phishing emails with Microsoft Word attachments to deploy malware.

- Key Techniques:

- - Spear-Phishing: Targeted attempt to steal sensitive information such as account credentials by masquerading as a trustworthy source.

- Impact: Caused a power outage for approximately 230,000 people in Ukraine.

Task 3: Understanding Attacker Motivations

3.1 Motivations for Attacking ICS/SCADA Systems

- Financial Gain: Cybercriminals may disrupt services or steal information for ransom or sell sensitive data.

- Political or Ideological Goals: Hacktivists or nation-states may target critical infrastructure to make political statements or weaken adversaries.

- Sabotage and Espionage: Industrial espionage can provide attackers' global competitive advantages or destabilize victimsâ operations.

Key Insight: Understanding motivations helps in preparing and implementing more effective defense strategies.

Lab 14: Introduction to Modbus Protocol

Objectives

- Understand the basic structure and functionality of the Modbus protocol.

- Learn to analyze Modbus packet structures including function codes and addresses.

- Use a Modbus simulator to query a test device.

- Document successful read and write operations using Modbus.

Prerequisites

- Basic understanding of network communication protocols.

- Familiarity with hexadecimal and binary notation.

- Access to a computer with internet connectivity.

- Pre-installed open-source Modbus simulator (e.g., Simply Modbus TCP Client).

Lab Tasks

Task 1: Study Modbus Packet Structure

1.1 Understand Modbus Fundamentals

- Modbus Protocol: A communication protocol developed by Modicon (now Schneider Electric) for industrial environments. It is used for transmitting information over serial lines between electronic devices.

- RTU and TCP Variants: Modbus operates using two primary types: RTU (Remote Terminal Unit) and TCP/IP. This lab focuses on Modbus TCP.

- Packet Structure: Composed of a unique address, function code, data, and CRC (Cyclic Redundancy Check).

1.2 Identify Function Codes and Addresses

- Function Codes: Specify the operation to be performed. Common codes include:

  - 0x03 - Read Holding Registers

  - 0x06 - Write Single Register

  - 0x10 - Write Multiple Registers

- Device Address: Identifies the target device for data exchange.

Task 2: Use a Modbus Simulator to Query a Test Device

2.1 Setup Modbus Simulator

- Download and install an open-source Modbus simulator, such as Simply Modbus TCP Client.

- Configure connection settings:

  - IP Address: Enter the IP of the Modbus server (e.g., 127.0.0.1 for local simulation).

  - Port: Default port is 502 for Modbus TCP.

2.2 Perform Read Operation

- Configure the read operation to gather data from the test device.

# Command to read holding register from address 40001

Function Code: 0x03

Starting Address: 40001

Number of Registers: 2

- Execute the command and review the response for accuracy.

Example Response:

Response Example: [01][03][04][1234][5678][CRC]

2.3 Perform Write Operation

- Set up a write command to modify a value at a specified register.

# Command to write a value to a single register at address 40001

Function Code: 0x06

Register Address: 40001

Value to Write: 000A

- Execute the command and document the success response.

Example Response:

Response Example: [01][06][4001][000A][CRC]

Task 3: Document Successful Read/Write Operations

- Log Communication: Record details of each operation including timestamps, function codes, and data exchanged.

- Verify Integrity: Ensure CRC checks provide confirmation of data integrity.

Lab 19: Patch Management and Updates in Industrial Control Systems (ICS)

Objectives

- Understand patch management in Industrial Control Systems (ICS)

- Learn to identify devices and systems in need of updates

- Develop strategies for safely applying patches to minimize disruptions

- Explore open-source tools for patch management

Prerequisites

- Basic knowledge of Industrial Control Systems

- Understanding of network and system security principles

- Access to a test ICS environment

- Basic familiarity with Linux command line

Lab Tasks

Task 1: Identify Devices or OS that Need Patches

Step 1: Inventory ICS Components

- Objective: Document and review all devices, operating systems, and software in your ICS environment.

- Tools: Use open-source tools like Nmap to scan your network for devices.

- Command Example:

- nmap -sP 192.168.1.0/24

- This command will provide a list of active IP addresses on your network.

Step 2: Check System and Software Versions

- Objective: Identify the current software and firmware versions running on each device.

- Review logs and system information to document software versions for comparison with known vulnerabilities.

Task 2: Check Official Vendor Patch Advisories

Step 1: Access Vendor Advisory Sites

- Visit official vendor websites for your ICS hardware and software.

- Identify any announcements related to patches, updates, or security vulnerabilities.

Step 2: Analyze Advisory Information

- List critical updates and their severity based on published CVEs (Common Vulnerabilities and Exposures).

Task 3: Discuss Scheduling Strategies for Safe Patching

Step 1: Develop a Patching Plan

- Objective: Create a plan that prioritizes patch application based on the criticality and role of each device.

- Consider downtime, network traffic loads, and operational requirements to minimize impact.

Step 2: Implement a Patching Schedule

- Schedule updates during non-peak hours or planned maintenance windows using tools like Ansible for automation.

- Command Example:

- ansible-playbook update_ics.yml -i inventory

- This command runs an Ansible playbook that automates the update process across selected ICS devices.

Key Concepts

- Patch Management: The process of managing software updates and patches intended to correct vulnerabilities and optimize functionality.

- Vendor Advisory: Notifications from vendors about updates, including security patches or feature enhancements.

- Downtime Management: Strategically planning updates to minimize disruption to operations.

Example & Case Study

Case Study: Water Treatment Facility

- A water treatment facility implements a biannual patch management plan.

- Challenges include maintaining continuous operation, which requires precise scheduling and aggressive testing before deployment.

Lab 20: Firmware Basics for ICS Devices

Objectives

- Understand the concept of firmware within Industrial Control Systems (ICS) devices.

- Identify and differentiate between different types of firmware used in ICS devices like PLCs (Programmable Logic Controllers).

- Explore basic procedures for firmware management, focusing on updating and securing firmware to maintain device integrity.

Prerequisites

- Basic understanding of Industrial Control Systems (ICS) and their components.

- Fundamental knowledge of networking concepts and device management.

- Access to a simulated environment containing a PLC or a hardware simulator (e.g., OpenPLC).

Lab Tasks

Task 1: Identify Current Firmware Version of a Simulated PLC

Step 1.1: Set Up a Simulated PLC Environment

- Tools Required: OpenPLC or a similar PLC simulation tool.

- Install OpenPLC using the following command in a Linux-based terminal:

- sudo apt-get update

- sudo apt-get install openplc

- Configure OpenPLC according to your system specifications.

Step 1.2: Access the PLC's Management Interface

- Open the OpenPLC web management interface via a web browser.

- Default URL (may vary based on installation): http://localhost:8080

Step 1.3: Locate Firmware Information

- Navigate to the "System Information" section.

- Identify and record the current firmware version displayed in the interface.

Task 2: Research Potential Firmware Vulnerabilities

Step 2.1: Understand Common Vulnerabilities

- Key Terms: Firmware vulnerabilities, exploits, updates.

- Research typical firmware vulnerabilities affecting ICS devices (e.g., buffer overflows, outdated libraries).

Step 2.2: Case Study Exploration

- Explore a case study on a known vulnerability affecting ICS devices, such as Stuxnet.

- Discuss how outdated firmware played a role in the exploit and what mitigations were introduced.

Task 3: Outline Steps to Perform a Firmware Update

Step 3.1: Verify Firmware Compatibility

- Check the manufacturer's website or documentation for the latest firmware version for the PLC model.

- Ensure compatibility with the existing system setup (hardware and software).

Step 3.2: Download the Latest Firmware

- Download the latest firmware version from the official manufacturerâs support portal.

Step 3.3: Execute a Firmware Update (Theoretical)

- Back up the PLC configuration and settings to prevent data loss during the update process.

- Follow these theoretical update steps:

  - Connect the PLC to the network via an Ethernet connection.

  - Access the PLC's firmware update module through the management interface.

  - Upload the downloaded firmware file.

  - Initiate the update process and wait for confirmation that the update is complete.

Step 3.4: Document the Update Process

- Note each step taken during the firmware update.

- Record the new firmware version for future reference and audits.

Lab Name: 21. Simple Intrusion Detection with Snort/Suricata

Objectives

- Understand the basic installation and configuration of Snort or Suricata

- Configure simple rules to detect specific network traffic patterns

- Simulate network traffic to test the efficacy of intrusion detection rules

Prerequisites

- Basic understanding of network protocols

- Familiarity with command-line interface

- A system with root or administrative access for installing and configuring software

- Internet access for downloading required open-source tools

Lab Tasks

Task 1: Install Snort/Suricata in a Lab Environment

1.1 Choose Your IDPS (Intrusion Detection and Prevention System)

Before starting, decide whether to use Snort or Suricata. Both are popular open-source intrusion detection systems.

1.2 Install Snort

Ubuntu/Debian Systems:

sudo apt update

sudo apt install snort

Red Hat/CentOS Systems:

sudo yum update

sudo yum install snort

1.3 Install Suricata

Ubuntu/Debian Systems:

sudo apt update

sudo apt install suricata

Red Hat/CentOS Systems:

sudo yum update

sudo yum install suricata

Task 2: Configure a Basic Rule to Detect Specific Modbus Traffic

2.1 Understand Snort/Suricata Rule Structure

- Snort and Suricata rules typically follow a format: (action) (protocol),(source),(destination),(msg).

2.2 Create a Simple Rule

Here, weâll create a rule to detect a specific pattern in Modbus traffic, a protocol used in industrial control systems.

For Snort, create a new rule in the local rules file (e.g., /etc/snort/rules/local.rules):

alert tcp any any -> any 502 (msg: "Modbus Traffic Detected"; sid:1000001; rev:1;)

For Suricata, the rule can be added to the default rule file or a custom user file (e.g., /etc/suricata/rules/local.rules):

alert tcp any any -> any 502 (msg: "Modbus Traffic Detected"; sid:1000001; rev:1;)

2.3 Reload the Configurations

For Snort:

sudo snort -T -c /etc/snort/snort.conf

For Suricata:

sudo suricata -T -c /etc/suricata/suricata.yaml

Task 3: Generate Traffic to Trigger the Rule

3.1 Use a Traffic Generator

You can use a tool like nmap to generate the Modbus traffic.

- Install nmap if it's not already installed:

- sudo apt install nmap

3.2 Simulate Modbus Traffic

Run the following command to simulate network traffic to a Modbus TCP port:

nmap -p 502 <target-ip>

3.3 Check the Alert Logs

- For Snort: Check alert logs at /var/log/snort/alert.

- For Suricata: Check logs at /var/log/suricata/fast.log.

Lab 22: ICS/SCADA Logging and SIEM Basics

Objectives

- Understand the fundamental concepts of logging in Industrial Control Systems (ICS) and Supervisory Control and Data Acquisition (SCADA) environments.

- Identify key log sources within ICS systems such as Programmable Logic Controllers (PLC) and Human Machine Interfaces (HMI).

- Learn to forward logs to a centralized server or Security Information and Event Management (SIEM) system.

- Analyze sample log entries to detect suspicious behavior.

Prerequisites

- Basic knowledge of ICS/SCADA systems.

- Understanding of network protocols and basic cybersecurity concepts.

- Access to a Linux environment with administrative privileges.

- Familiarity with SIEM tools (e.g., OSSEC, ELK Stack).

Lab Tasks

Task 1: Identify Key Log Sources in ICS Systems

- Objective: Recognize different log sources in ICS/SCADA environments.

Subtasks:

  - PLC Logs:

    - Locate and access log files generated by PLCs.

    - Understand typical log file formats and information contained within (e.g., operation status, error messages).

    - Example: "A PLC log entry might indicate a change of state in a physical input or a runtime error."

  - HMI Logs:

    - Identify HMI devices in the SCADA environment.

    - Collect logs showing interactions, status changes, and error messages.

    - Technical Terms: "HMI", "interface interactions", "system status".

Task 2: Forward Logs to a Centralized Server or SIEM

- Objective: Implement log forwarding to enhance security monitoring.

Subtasks:

  - Log Forwarding with Syslog:

    - Install and configure the syslog service.

    - Configure clients to forward logs to a central syslog server.

    - Code Example:

  - # Install Rsyslog on Linux

  - sudo apt update

  - sudo apt install rsyslog

  - # Configure the client (/etc/rsyslog.conf)

  - *.* @centralized.server.ip:514

  - # Restart the service

  - sudo systemctl restart rsyslog

  - Integration with SIEM (Conceptual):

    - Discuss how collected logs can be routed to a SIEM platform for advanced analysis and alerting.

    - Highlight open-source SIEM solutions like OSSEC or the ELK Stack.

Task 3: Examine Sample Log Entries for Suspicious Behavior

- Objective: Analyze ICS log entries to identify potential security incidents.

Subtasks:

  - Log Entry Analysis:

    - Review example logs to detect patterns indicating unauthorized access or anomalies.

    - Example Log Entry:

  - 2023-11-01 10:05:13 [PLC01] User: Admin Access: Granted SourceIP: 192.168.1.100

  - Detecting Suspicious Patterns:

    - Use tools like grep or SIEM search functionalities to identify signs of compromise.

    - Code Example (using grep):

  - # Search for failed access attempts in PLC logs

grep "Access: Denied" plc_logs.txt

Lab 23: Basic ICS/SCADA Asset Inventory

Objectives

- Understand the components of an ICS/SCADA system and their roles.

- Learn how to perform a basic asset inventory of hardware components like PLCs, RTUs, and sensors.

- Catalog software elements such as HMIs and engineering workstations.

- Document firmware/OS versions.

- Identify and map basic network interconnections.

Prerequisites

- Basic understanding of ICS/SCADA systems.

- Familiarity with network configurations and terminologies.

- Access to an ICS/SCADA environment or simulation.

Lab Tasks

Task 1: Hardware Inventory

- Identify Components

  - Identify and list all physical components such as Programmable Logic Controllers (PLCs), Remote Terminal Units (RTUs), and sensors.

  - Document the location and physical condition of each device.

  - Example: Use open-source tools like Nmap to detect devices on the network.

- Document Specifications

  - Record firmware versions, serial numbers, and any associated documentation.

  - Highlight: It's crucial to know the firmware versions for maintenance and security updates.

- Subtask Example

  - Use Nmap to find PLCs on your network:

  - nmap -p 502 --script modbus-discover <IP_range>

  - Record findings in a CSV file for further analysis.

Task 2: Software Inventory

- Identify Software Components

  - List software components, including Human-Machine Interfaces (HMIs) and engineering workstations.

  - Document software versions and license details.

- Catalog System Hostnames and IP Addresses

  - Use open-source network tools to map connections and identify software elements across the network.

  - Highlight: Keep a detailed log for system changes and updates.

- Subtask Example

  - Use tools such as Apache's Network Discovery to identify networked hosts:

  - nmap -sn <network/subnet>

Task 3: Network Interconnections Inventory

- Map Network Configuration

  - Draw or document the network layout, including connections between devices.

  - Use tools like Cacti or Zabbix for network monitoring and map validation.

- Identify Critical Interconnections

  - Pinpoint essential connections that maintain system operation.

  - Highlight: Knowing critical paths helps in securing them against potential disruptions.

- Subtask Example

  - Using Wireshark to capture and analyze traffic:

  - wireshark -i <network_interface>

Case Study Example

Participants will reference a simulated ICS/SCADA system, observing the network scan process performed using Wireshark and Nmap. They will inventory system components and document their findings through practical examples.

Lab 24: ICS/SCADA Risk Assessment 101

Objectives

- Understand the basics of ICS/SCADA systems and their unique risks.

- Learn to identify potential threats such as unauthorized access and malware.

- Determine and assess the possible impacts of threats to ICS, including downtime and safety concerns.

- Rank risks based on their likelihood and potential impact on the system.

- Conduct a practical risk assessment on a simulated ICS/SCADA environment using open-source tools.

Prerequisites

- Basic understanding of network security and industrial control systems.

- Familiarity with cybersecurity concepts.

- Access to a computer with internet connectivity.

- Installation of open-source software tools like Wireshark and Nmap.

Lab Tasks

Task 1: Introduction to ICS/SCADA Systems

1.1 Understanding ICS/SCADA

- Industrial Control Systems (ICS) are used for controlling industrial processes.

- Supervisory Control and Data Acquisition (SCADA) is a subset of ICS used for remote monitoring and control.

1.2 Unique Risks

- Unlike IT systems, ICS/SCADA focuses on availability and safety.

- They are often targeted for disruption, causing physical or economic harm.

Task 2: Threat Identification

2.1 Common Threats

- Unauthorized Access: External or internal threat actors gaining unauthorized access to control systems.

- Malware: Malicious software that can disrupt operations or steal data.

Example Case Study:

- The 2010 Stuxnet worm targeted Iranian nuclear facilities, highlighting ICS vulnerabilities.

Task 3: Risk Assessment Process

3.1 Determine Possible Impacts

- Downtime: Loss of control and operational downtime.

- Safety Hazards: Risk of physical harm to operators and facilities.

3.2 Ranking Risks

- Use a qualitative matrix to rank risks based on:

  - Likelihood: Frequency of occurrence.

  - Impact: Severity of potential consequences.

3.3 Example Matrix

| Risk Type          | Likelihood | Impact  | Rank |

|--------------------|------------|---------|------|

| Unauthorized Access| High       | High    | 1    |

| Malware Infection  | Medium     | Medium  | 2    |

Task 4: Practical Lab Exercise

4.1 Setting Up the Simulated Environment

- Install Nmap for network scanning.

- Install Wireshark for packet analysis.

4.2 Conducting a Basic Risk Assessment

- Step 1: Use Nmap to perform a network scan.

nmap -sS 192.168.0.0/24

- Step 2: Use Wireshark to capture packet data.

wireshark &

- Step 3: Analyze the results to identify potential vulnerabilities.

4.3 Document Findings

- Create a report summarizing findings, including identified threats, potential impacts, and suggested mitigations.

Lab 25: Incident Response Basics for ICS/SCADA

Objectives

- Understand the roles and responsibilities in an ICS incident response team.

- Develop a basic incident response plan tailored for ICS/SCADA environments.

- Identify and implement containment strategies to mitigate incidents effectively.

Prerequisites

- Basic understanding of Industrial Control Systems (ICS) and SCADA.

- Familiarity with cybersecurity concepts and incident response frameworks.

- Access to an open source virtual environment for simulation.

Task 1: Outline Roles and Responsibilities in an ICS Incident

Step 1: Identify Key Roles

- Incident Response Leader: Coordinates and leads the incident response efforts.

- SCADA Engineer: Provides technical expertise on the SCADA system.

- IT Security Analyst: Analyzes the security breach and suggests mitigation strategies.

- Communications Officer: Handles communications internally and externally.

Step 2: Document Responsibilities

- Incident Response Leader:

  - Coordinate incident response activities.

  - Report updates to management and stakeholders.

- SCADA Engineer:

  - Analyze system data for anomalies.

  - Initiate system shutdown if necessary.

- IT Security Analyst:

  - Perform root cause analysis.

  - Implement security controls to prevent reoccurrence.

- Communications Officer:

  - Ensure clear and concise communication.

  - Manage media inquiries and press releases.

Task 2: Create a Short Incident Response Plan Template

Step 1: Develop an Incident Response Plan Template

## Incident Response Plan For ICS/SCADA

### 1. Introduction

- Purpose

- Scope

### 2. Incident Detection

- Monitoring tools

- Anomaly detection indicators

### 3. Incident Notification

- Reporting mechanisms

- Contact details

### 4. Containment Strategy

- Immediate actions

- Isolation processes

### 5. Eradication Plans

- Removing threats

- System recovery steps

### 6. Recovery Actions

- Reintroduction of systems to the network

- Validation steps

### 7. Lessons Learned

- Post-incident review

- Improvement strategies

Step 2: Customize to Specific Environment

- Adapt the template according to your specific ICS/SCADA environment. Ensure it includes specific contacts and customized containment strategies.

Task 3: Identify Containment Strategies

Step 1: Implement Isolation Techniques

- Isolating Infected Hosts:

  - Disconnect compromised devices from the network.

  - Use VLAN configuration to prevent further spread.

# Example command for VLAN isolation

vconfig add eth0 100

ifconfig eth0.100 192.168.1.1 netmask 255.255.255.0

Step 2: Deploy Network Segmentation

- Define zones for critical and non-critical assets to minimize risks.

- Ensure communication between zones is controlled and monitored.

Step 3: Installation of Intrusion Detection Systems (IDS)

- Use open source IDS like Snort to monitor network traffic for suspicious activities.

# Example to install and configure Snort on a Unix-based system

sudo apt-get update

sudo apt-get install snort

snort -dev -l /var/log/snort -h 192.168.1.0/24 -c /etc/snort/snort.conf