Lab 26: ICS/SCADA Backup and Recovery Fundamentals

Objectives

- Understand the fundamentals of backup and recovery in ICS/SCADA systems.

- Identify and prioritize critical system components for backup.

- Explore different backup strategies suitable for ICS/SCADA environments.

- Create a simple backup schedule using open-source tools.

Prerequisites

- Basic understanding of ICS/SCADA systems and their components.

- Familiarity with network configurations and cybersecurity principles.

- Access to a virtualized lab environment with open-source tools such as rsync and cron.

Lab Tasks

Task 1: Identify Critical System Components for Backup

- Overview of ICS/SCADA Components

  - Human-Machine Interface (HMI): Interfaces allowing human operators to interact with the control system.

  - Programmable Logic Controllers (PLCs): Hardware used to monitor and control industrial processes.

- Identification Process:

  - HMI Configurations:

    - Locate and document current HMI configuration files.

    - Example: /var/scada/hmi/config.ini

  - PLC Programs:

    - Identify where PLC programs are stored.

    - Data Path Example: /var/scada/plc/

Task 2: Discuss Offline/Online Backup Strategies

- Offline Backup:

  - Definition: Backups stored on disconnected storage media, reducing exposure to network-based threats.

  - Benefits: High security, protects against ransomware attacks.

  - Example: Daily backup using an external hard drive.

- Online Backup:

  - Definition: Backups stored on network-connected devices or cloud services for easy access and retrieval.

  - Benefits: Real-time data protection, easier recovery.

  - Example: Utilizing tools like Nextcloud for cloud-based storage.

Task 3: Create a Simple Backup Schedule

- Using rsync for Backup:

  - Installation (if necessary)

    - sudo apt-get update

    - sudo apt-get install rsync

  - Basic Backup Command:

    - rsync -avz /var/scada/ /backup/scada/

  - Explanation:

    - -a stands for archive mode, preserving permissions.

    - -v enables verbose output.

    - -z compresses the files during transfer.

- Automating Backup with Cron Jobs:

  - Set up Cron Job:

    - Edit crontab file:

    - crontab -e

    - Schedule a daily backup at 2 AM:

    - 0 2 * * * rsync -avz /var/scada/ /backup/scada/

  - Explanation:

    - The cron schedule notation: 0 2 * * * specifies 2:00 AM every day.

Lab 27: Basic ICS/SCADA Change Management

Objectives

- Understand the fundamentals of change management in ICS/SCADA environments.

- Learn to define a basic change management procedure.

- Document a hypothetical change and track it using a log or ticketing system.

Prerequisites

- Basic understanding of ICS/SCADA systems.

- Familiarity with version control systems and open-source tools.

- Access to a computer with internet connectivity.

Overview

Industrial Control Systems (ICS) and Supervisory Control and Data Acquisition (SCADA) systems are crucial for monitoring and controlling industrial processes. Change management ensures that changes are made systematically, minimizing risks and maintaining system integrity.

Lab Tasks

Task 1: Define a Change Management Procedure

- Step 1: Understand Change Management Principles

  - Change management involves procedures for handling changes systematically.

  - Key Components:

    - Change Requests: Initiating the change.

    - Approvals: Obtaining necessary permissions.

    - Testing: Verifying changes before implementation.

- Step 2: Create Your Change Management Procedure Document

  - Use the following template to draft your procedure:

## Change Management Procedure

### 1. Initiating Change Requests

- Identify the need for change.

- Document the change request using a standard format.

### 2. Approval Process

- Who is responsible for approving changes?

- Define approval levels based on change impact.

### 3. Testing Changes

- Procedures for testing changes in a non-production environment.

- Criteria for successful testing.

### 4. Implementation and Review

- Steps for implementing changes.

- Conduct a post-implementation review.

Task 2: Document a Hypothetical ICS Change

- Scenario: Company X wishes to update the HMI screens for clarity and efficiency.

- Step 1: Create a Change Request Document

  - Use the following example to build your change request:

## Change Request Document

### Requestor

- Name: [Jane Doe]

- Role: [SCADA Engineer]

### Change Description

- Update HMI screens to enhance user interface and operation clarity.

### Reason for Change

- Improve operator efficiency and reduce errors.

### Impact Analysis

- Minimal impact; changes highly user-interface centric.

### Testing Plan

- Conduct testing in a simulated environment after design approval.

### Approval Status

- Awaiting approvals from HMI Design Team Lead and Operations Manager.

Task 3: Track the Change in a Log or Ticketing System

- Step 1: Choose an Open-Source Ticketing System

  - Recommended Tool: OTRS (Open-source Ticket Request System).

- Step 2: Log the Change Request

  - Log a ticket with necessary details from the change request document.

## OTRS Ticket Entry

- **Ticket ID**: [TICK-20231001]

- **Title**: [HMI Screen Update Request]

- **Description**: [As detailed in Change Request Document]

- **Current Status**: [Pending Approval]

- **Approval History**: [Document timeline and notes]

- Step 3: Monitor the Change

  - Track the status and documentation related to the change.

Lab Name: Detecting Unauthorized Devices

Objectives

- Understand how to use network scanning tools to identify authorized and unauthorized devices.

- Learn to establish a network baseline and identify deviations.

- Implement physical checks to locate and mitigate unauthorized devices.

Prerequisites

- Basic understanding of networking concepts.

- Access to a network environment for testing purposes.

- Installation of open-source network scanning tools such as nmap.

Tasks and Steps

Task 1: Network Scanning for Baseline Devices

Step 1: Install nmap

- On Linux:

- sudo apt update

- sudo apt install nmap

- On macOS (using Homebrew):

- brew install nmap

Step 2: Conduct a Network Baseline Scan

- Command: Perform a scan to identify all devices within your local network.

- sudo nmap -sn 192.168.1.0/24

- Explanation: This command performs a ping scan over the subnet to list all active devices. Replace 192.168.1.0/24 with the actual network range.

Step 3: Document Known Devices

- Task: Record the IP addresses, MAC addresses, and hostnames of each known device.

- Tip: Use a spreadsheet or a network management tool to track your devices efficiently.

Task 2: Detecting Unknown or Unauthorized Hosts

### Step 1: Regular Network Scans

- Command: Schedule regular scans to detect any changes in the network.

- sudo nmap -sn 192.168.1.0/24

- Explanation: Compare these scans with your baseline to identify any new or unauthorized devices.

Step 2: Analyze Scan Results

- Example: Suppose your baseline scan showed 10 devices, and a new scan shows 12. Identify the 2 additional devices.

- Action: Investigate unknown devices by correlating their MAC address with vendor lists or physical checks.

Task 3: Physical Checks for Rogue Devices

Step 1: Physical Network Inspection

- Concept: Manually inspect physical connections if a device's origin is unclear from the scan data.

Step 2: Utilize Network Management Tools

- Example: Tools like Wireshark can help analyze packets and reveal unauthorized data traffic.

Key Concepts

- Network Baseline: A known state of network devices against which future scans are compared to detect anomalies.

- Unauthorized Devices: Devices not recognized or approved to join the network, potentially posing a security risk.

Example Case Study

Scenario: An organization routinely finds unauthorized devices on their network, leading to data breaches.

Solution: By conducting regular nmap scans and maintaining a baseline, they reduced unauthorized access by 80%.

Lab Name: 29. ICS/SCADA Physical Device Hardening Basics

Objectives:

- Understand the importance of physical security for ICS/SCADA devices.

- Learn methods and tools to secure physical devices.

- Explore environmental factors that affect device security and operational integrity.

Prerequisites:

- Basic understanding of ICS/SCADA systems.

- Familiarity with network security basics.

- Access to a test ICS/SCADA setup for hands-on practice.

Introduction

In industrial environments, ensuring the safety and security of ICS/SCADA devices is critical. These devices control essential infrastructure and require robust protection against both physical and cyber threats. This lab will guide you through basic strategies to physically harden these devices against potential threats.

Lab Tasks

Task 1: Review Device Casing and Enclosure Security

Subtask 1.1: Assess Physical Enclosures

- Objective: Identify the security level of current device enclosures.

- Action Steps:

  - Evaluate the material and construction of device enclosures.

  - Inspect for existing damage or vulnerabilities.

  - Ensure each enclosure applies the principle of redundancy to avoid single points of failure.

Subtask 1.2: Implement Security Enhancements

- Objective: Add security features to existing enclosures.

- Action Steps:

  - Install tamper-evident seals to detect unauthorized access.

  - Deploy physical hardware locks compatible with your enclosures.

  - Implement access control measures such as biometric scanners or card access systems.

Example:

- Use a security seal tape to leave a detectable mark when peeled.

Task 2: Environmental Factors Impact Analysis

Subtask 2.1: Analyze Environmental Impact

- Objective: Determine how environmental factors affect device security and function.

- Action Steps:

  - Evaluate current enclosure resilience to dust, heat, and vibration.

  - Deploy sensors to monitor environmental conditions critical to device operation.

Key Concepts:

- Dust and Debris: Can lead to device overheating and failure.

- Heat: Extreme temperatures can affect device integrity.

- Vibration: May disrupt connections and circuitry.

Subtask 2.2: Mitigation Techniques

- Objective: Apply methods to reduce environmental risks.

- Action Steps:

  - Seal all entry points with dust-proof gaskets.

  - Use thermal management solutionsâsuch as heat sinks and fans.

  - Install vibration dampening mounts where necessary.

Case Study:

- XYZ Manufacturing reduced device failures by 40% by implementing vibration dampening solutions.

Task 3: Documentation and Policy Development

Subtask 3.1: Create Security Protocols

- Objective: Establish a comprehensive set of security protocols.

- Action Steps:

  - Draft policies for regular security audits and maintenance checks.

  - Create incident response plans for physical breaches or environmental failures.

Key Concept:

- Regular Audits: Ensure ongoing compliance with security protocols.

Lab 31: Basic Whitelisting in ICS Environments

Objectives

- Understand the concept of whitelisting in Industrial Control Systems (ICS).

- Identify critical ICS software components for whitelisting.

- Implement a basic whitelisting policy using open-source tools.

- Validate the whitelisting by attempting to execute unauthorized software.

Prerequisites

- Basic understanding of Industrial Control Systems (ICS) and their operational needs.

- Familiarity with Linux command-line interface.

- An environment consisting of a Linux-based system (such as a virtual machine) with root access for testing purposes.

Tasks

Task 1: Identify Critical ICS Software for Whitelisting

Subtask 1.1: Understand the ICS Environment

- Research the ICS Components:
Understand the primary software components within your ICS environment that require guaranteed uptime and security.

Example: In a power grid system, these might include SCADA systems, RTUs (Remote Terminal Units), and HMIs (Human-Machine Interfaces).

Subtask 1.2: Create a List of Critical Software

- Document Essential Software:
List out all the critical software applications which are essential for ICS operations.

Example: SCADA application (scada-app), Database server (ics-db).

Task 2: Implement a Simple Whitelisting Policy

Subtask 2.1: Install and Configure Open Source Whitelisting Tool

- Select an Open Source Tool:
Consider using AppArmor, as it provides a framework for service confinement through security policies.

- Install AppArmor:

- sudo apt-get update

- sudo apt-get install apparmor apparmor-utils

- Enable AppArmor:

- sudo systemctl enable apparmor

- sudo systemctl start apparmor

Subtask 2.2: Create a Whitelist Profile

- Generate Profile for Critical Software:

  - Use aa-genprof tool to create profiles for critical applications.

- sudo aa-genprof /path/to/scada-app

- Activate the Profile:

  - Set the profile to enforce mode:

- sudo aa-enforce /etc/apparmor.d/usr.bin.scada-app

Task 3: Validate Whitelisting

Subtask 3.1: Test Whitelisted Application

- Run the Whitelisted Software:
Ensure that your ICS applications (e.g., scada-app) are running without interruption.

Example Command:

/path/to/scada-app

Subtask 3.2: Attempt to Run Unauthorized Software

- Execute Unauthorized Software:
Run any non-whitelisted software (e.g., a generic text editor not part of ICS operations).

Example Command:

/usr/bin/gedit

- Observe the Output:
Ensure that the unauthorized execution is blocked by AppArmor rules.

Lab 32: Introduction to ICS Security Standards

Objectives

- Understand the basic principles of ICS (Industrial Control Systems) Security.

- Review and summarize the NIST SP 800-82 guidelines.

- Identify major takeaways relevant to an ICS setup.

- Compare key differences between ICS and IT-centric security standards.

Prerequisites

- Basic understanding of ICS and network security.

- Access to an Ubuntu or CentOS Linux environment for executing example commands.

Task 1: Review NIST SP 800-82 Guidelines

1.1 Overview

- NIST SP 800-82 provides guidelines for securing ICS.

- It focuses on risk management, security controls, and architecture.

1.2 Key Concepts

- ICS Security Risks: Understanding threats specific to ICS environments, such as physical threats and cyber-attacks.

- Security Controls: NIST outlines controls tailored for ICS to mitigate identified risks.

1.3 Summary Review

- Read the NIST SP 800-82 document: NIST SP 800-82 Rev. 2

- Highlight sections on security issues distinct to ICS.

Task 2: List Major Takeaways Relevant to ICS Setup

2.1 Identifying Key Takeaways

- Focus on tailored security strategies for ICS setups, stressing:

  - Physical Security

  - Network Segmentation

  - Patch Management

2.2 Example

- Example Scenarios:

  - Case Study: Implementing a firewall within a utility company's SCADA network to separate the business network from process networks.

2.3 Writing Down Takeaways

- Open a text editor and document the strategies. For instance:

- nano ics_takeaways.txt

- Example Content:

- - Implement strict physical access controls.

- - Use network segmentation to protect ICS from enterprise-level risks.

- - Regularly update systems and firmware to patch known vulnerabilities.

Task 3: Compare Differences to IT-Centric Standards

3.1 Study of IT-Centric Standards

- Examples include ISO/IEC 27001, focusing primarily on:

  - Data Protection

  - Incident Response

3.2 Key Differences

- Real-Time Processing: Unlike IT systems, ICS require consistent real-time data processing.

- Legacy Systems: ICS often integrate with legacy hardware and software, posing unique challenges.

3.3 Detailed Comparison

- Create a comparison table using Markdown:

- | Feature                    | ICS Security (NIST SP 800-82) | IT Security (ISO/IEC 27001) |

- |----------------------------|------------------------------|-----------------------------|

- | Real-time Requirements     | Integral                     | Substantial but less critical|

- | Legacy Systems             | Prevalent                    | Less often considered       |

- | Physical Security          | Emphasized                   | Moderate emphasis           |

- Command to create and edit a Markdown file:

nano ics_vs_it.md

Lab 33: ICS/SCADA Logging with Syslog

Objectives

- Understand and configure logging for ICS/SCADA environments using Syslog.

- Set up a Syslog server and forward logs from an ICS or PLC simulator.

- Validate log transmission and storage integrity.

Prerequisites

- Basic understanding of ICS/SCADA systems.

- Familiarity with Linux command line operations.

- Access to a virtualized environment or hardware to install the required simulation tools.

- Linux-based system for Syslog server and ICS simulator setup.

Lab Tasks

Task 1: Install and Configure a Syslog Server

- Install Syslog Server

  - Use a Linux-based distribution. For this lab, we'll use Ubuntu.

- sudo apt-get update

- sudo apt-get install rsyslog

- Configure rsyslog

  - Edit the rsyslog configuration file to accept remote logs.

- sudo nano /etc/rsyslog.conf

  - Uncomment or add the following lines to enable reception of TCP and UDP messages.

- module(load="imudp")

- input(type="imudp" port="514")

- module(load="imtcp")

- input(type="imtcp" port="514")

- Restart rsyslog service

  - Apply changes by restarting the rsyslog service.

- sudo systemctl restart rsyslog

Task 2: Configure an ICS Host or PLC Simulator for Syslog Forwarding

- Install a PLC Simulator

  - Use an open-source PLC simulator such as MATPLC.

- sudo apt-get install matiec

- Configure the Simulator to Forward Logs

  - Check the simulator's documentation for Syslog support.

  - Example configuration might involve editing a config file to add a Syslog server address.

  - Add an entry similar to the following in the simulator's configuration:

- syslog_address=192.168.1.100:514

Task 3: Verify Logs Reception

- Monitor Incoming Logs on Syslog Server

  - Tail the log file to verify incoming logs.

- sudo tail -f /var/log/syslog

- Generate Test Logs from Simulator

  - Execute a command or action on the ICS simulator that generates log entries.

- Validation

  - Confirm the logs appear in the syslog output and examine them for expected entries.

Conclusion

- You have set up a Syslog server and configured an ICS/SCADA simulator to forward logs.

- You should now understand the flow of logs in an ICS/SCADA environment and the importance of logging for monitoring and security.

- The skills acquired here will help in maintaining the security and reliability of ICS environments through robust logging practices.

Key Concepts: Syslog, ICS/SCADA system, remote logging, network security.

Lab 34: Basic ICS Network Traffic Analysis

Objectives

- Understand and analyze the characteristics of network traffic in Industrial Control Systems (ICS).

- Identify and classify communication protocols such as Modbus and DNP3.

- Establish baseline bandwidth usage patterns for network traffic monitoring.

Prerequisites

- Basic understanding of networking concepts.

- Familiarity with ICS environments and protocols.

- Installation of Wireshark or any other relevant open-source packet analysis tool.

- Access to an ICS network environment or a simulated network.

Tools Required

- Wireshark: An open-source network protocol analyzer.

Tasks

Task 1: Capture Network Traffic During Normal ICS Operations

- Set Up Wireshark
Ensure Wireshark is installed on your analysis machine. Launch Wireshark.

- Select Network Interface

  - Choose the network interface connected to the ICS network.

  - Click on the interface to start capturing traffic.

Example:

sudo wireshark

- Capture Network Traffic

  - Leave the capture running while ICS operations proceed normally.

  - Aim for at least 15-20 minutes of traffic data to ensure you capture a diverse range of packets.

- Stop the Capture

  - Once you have sufficient data, click on the red square in the toolbar to stop the capture.

  - Save the traffic capture for analysis.

Task 2: Identify Protocol Types

- Filter by Protocols

  - Use Wireshark to filter packets by common ICS protocols such as Modbus and DNP3.

Example:

modbus || dnp3

- Analyze Protocol Characteristics

  - Review the Info column in Wireshark to get insights into the nature of the communication.

  - Note characteristics such as frequency, packet size, and interactions.

- Document Protocol Usage

  - Record the occurrence frequency of each protocol.

  - Mention any peculiarities noticed during the communication.

Task 3: Note Baseline Bandwidth Usage and Patterns

- Evaluate Bandwidth

  - Assess the data flow volume using the Statistics menu in Wireshark.

Steps:

  - Go to Statistics > IO Graphs.

  - Analyze the bandwidth using the visual statistics provided.

- Establish Baseline Patterns

  - Observe the traffic flow to determine normal bandwidth usage.

  - Identify any spikes or drops which are apart from regular patterns.

- Document Findings

  - Summarize the normal bandwidth consumption.

  - Highlight any deviations from expected patterns.

Lab Name: Introduction to ICS Forensics

Objectives

- Understand the unique aspects of forensic imaging in Industrial Control Systems (ICS).

- Identify critical data sources, such as logs and configuration files, in ICS environments.

- Explain chain-of-custody requirements specific to ICS incidents.

Prerequisites

- Basic understanding of Industrial Control Systems architecture.

- Familiarity with general forensic principles and methods.

- Access to a virtual or physical ICS environment for practice.

Tasks and Steps

Task 1: Understanding Forensic Imaging for ICS Devices

Step 1.1: Introduction to ICS Forensic Imaging

- Explain how forensic imaging in ICS differs from typical IT environments.

  - Highlight the importance of capturing both control system data and traditional IT data.

  - Discuss the challenges in non-intrusive data acquisition due to the operational nature of ICS.

Step 1.2: Considerations for Capturing ICS Data

- Discuss the types of data that must be captured in ICS:

  - Real-time process data: These are crucial for assessing system state during the incident.

  - System configurations: Understand the working and state of the PLC/SCADA systems.

Step 1.3: Tools for Imaging ICS Data

- Explain open-source tools that can be used for ICS data acquisition:

  - Example Tool: dd in Linux for disk imaging.

  - Example Command for Imaging:

  - dd if=/dev/sdX of=/path_to_output.img bs=4M status=progress

  - Highlight the need to preserve metadata.

Task 2: Identifying Important Data in ICS

Step 2.1: Collecting Logs and Configurations

- Logs: Explain the types of logs relevant to ICS:

  - Device logs from PLCs and SCADA systems.

  - Network traffic logs.

- Configurations: Importance of capturing configuration details:

  - Use of version control repositories to manage configurations.

Step 2.2: Automated Log Collection

- Introduce open-source tools like Syslog-ng or Logstash for log aggregation.

  - Example Logstash Configuration:

  - input {

  - file {

  - path => "/var/log/ics/*.log"

  - start_position => "beginning"

  - }

  - }

  - output {

  - elasticsearch {

  - hosts => ["http://localhost:9200"]

  - index => "ics-logs-%{+YYYY.MM.dd}"

  - }

  - }

Task 3: Chain-of-Custody in ICS

Step 3.1: Chain-of-Custody Principles

- Define key principles of chain-of-custody:

  - Documenting each step when evidence is handled.

  - The importance of maintaining a log of all individuals who interact with the data.

Step 3.2: Applying Chain-of-Custody in ICS Scenarios

- Discuss specific scenarios in ICS:

  - Handling evidence in environments with ongoing operations.

  - The role of digital signatures and hashing to ensure data integrity.

  - Example command for hashing a file with SHA-256:

sha256sum /path_to_output.img

Lab 36: Endpoint Security for ICS Workstations

Objectives

- Understand and implement basic endpoint security measures on an ICS workstation.

- Learn to enable basic antivirus or endpoint protection using open-source tools.

- Explore methods to restrict USB access.

- Document software inventory for better analysis and safety.

Prerequisites

- Basic understanding of Industrial Control Systems (ICS) and workstation management.

- Access to an ICS workstation for execution of the tasks.

- Administrative privileges on the workstation.

- Internet access to download open-source tools.

Task 1: Enable Basic Antivirus or Endpoint Protection

1.1 Install ClamAV

ClamAV is an open-source antivirus engine widely used for detecting malware.

Steps:

- Update Package List

Before installing new software, ensure you have the latest package information.

sudo apt-get update

- Install ClamAV

Run the following command to install ClamAV on your Linux workstation.

sudo apt-get install clamav clamav-daemon

- Update ClamAV Database

Ensure the virus database is up to date with the latest definitions.

sudo freshclam

- Scan the System

Perform a comprehensive scan of your ICS workstation.

sudo clamscan -r --bell -i /

Key Concept

- ClamAV is renowned for its ability to be integrated with mail providers and can be used in endpoint protection to mitigate potential threats.

Task 2: Restrict USB Access

2.1 Disable USB Ports using Modprobe

Using modprobe settings, we can restrict USB access effectively.

Steps:

- Create Backup

Always back up your current configuration.

sudo cp /etc/modprobe.d/blacklist.conf /etc/modprobe.d/blacklist.conf.backup

- Blacklist USB Storage

Modify the file to include the necessary line to disable USB storage.

echo "blacklist usb-storage" | sudo tee -a /etc/modprobe.d/blacklist.conf

- Update initramfs

Run the update so changes take effect.

sudo update-initramfs -u

Key Concept

- Blacklisting USB storage modules helps prevent unauthorized data transfer and protects sensitive industrial data.

Task 3: Document Software Inventory

3.1 Use dpkg-query Tool

For Linux systems, dpkg-query can be used to list all installed software, creating an easily accessible inventory.

Steps:

- List Installed Packages

Use the following command to generate a list of all installed packages.

dpkg-query -l > software-inventory.txt

- Review Inventory

Open the generated file to review the current software state.

less software-inventory.txt

Key Concept

- Maintaining a software inventory aids in managing software updates and verifying authorized applications.

Lab Name: ICS/SCADA Remote Update Procedure

Objectives:

- Understand the process of executing remote updates for ICS/SCADA systems.

- Learn to establish secure connections using VPN and appropriate firewall rules.

- Develop strategies for rolling back updates in case of failures.

Prerequisites:

- Basic understanding of ICS/SCADA systems and their architecture.

- Familiarity with network security concepts, particularly VPNs and firewall configuration.

- Access to an open-source VPN tool such as OpenVPN.

- Basic experience with command-line interface (CLI).

Lab Tasks:

Task 1: Create a Step-by-Step Update Plan for a Remote ICS Site

1.1 Assess the Existing System

- Identify all components within the ICS/SCADA system that will require updating.

- Document the software versions currently in use.

1.2 Develop a Detailed Update Plan

- Prioritize components based on their criticality in the network.

- Schedule updates during a maintenance window to minimize impact.

Example:

| Component                        | Priority | Scheduled Update Time |

|----------------------------------|----------|-----------------------|

| PLC Firmware                     | High     | 02:00 AM - 03:00 AM   |

| SCADA Monitoring Software        | Medium   | 03:00 AM - 04:00 AM   |

| Gateway Routers                  | Low      | 04:00 AM - 05:00 AM   |

- Ensure backup systems and data redundancy are in place.

Task 2: Emphasize Secure Connection (VPN, Firewall Rules)

2.1 Establish a Secure VPN Connection

- Use OpenVPN to set up a secure channel for remote update operations.

Installation on Ubuntu:

sudo apt update

sudo apt install openvpn

- Configure OpenVPN with a secure certificate and private key to ensure encrypted communications.

Example Config File:

client

dev tun

proto udp

remote your-vpn-server.com 1194

resolv-retry infinite

nobind

persist-key

persist-tun

ca ca.crt

cert client.crt

key client.key

cipher AES-256-CBC

comp-lzo

verb 3

2.2 Configure Firewall Rules

- Restrict all non-essential ports to secure the network perimeter.

- Allow only necessary traffic through the VPN.

Example using iptables:

iptables -A INPUT -p tcp --dport 1194 -j ACCEPT

iptables -A INPUT -m state --state ESTABLISHED,RELATED -j ACCEPT

iptables -A INPUT -j DROP

Task 3: Discuss Rollback Strategies if the Update Fails

3.1 Implement Backup and Recovery Procedures

- Conduct full backups of configurations and data before initiating updates.

Example with rsync:

rsync -av --progress /etc/config /backup-directory/config-backup

3.2 Develop a Rollback Plan

- Define precise rollback steps for each component.

- Prepare scripts to automate the rollback process where possible.

Shell Script Example:

#!/bin/bash

echo "Starting rollback..."

rsync -av --progress /backup-directory/config-backup /etc/config

if [[ $? -eq 0 ]]; then

echo "Rollback successful"

else

echo "Rollback failed"

fi

Lab 39: ICS/SCADA Configuration Management

Objectives

- Understand the importance of configuration management within ICS/SCADA systems.

- Learn to establish a version control system for PLC (Programmable Logic Controller) and HMI (Human-Machine Interface) configurations.

- Record configuration changes into a configuration management database (CMDB).

- Conduct periodic reviews to ensure configurations match the recorded versions.

Prerequisites

- Basic understanding of ICS/SCADA systems.

- Familiarity with version control systems such as Git.

- Fundamental knowledge of databases and how to interact with them.

- Tools required: Git, SQLite (or another open-source database), a text editor like Visual Studio Code.

Tasks

Task 1: Establish a Version Control System for PLC and HMI Configurations

1.1 Install Git

- Windows: Download and install Git from git-scm.com.

- Linux:

- sudo apt-get update

- sudo apt-get install git

1.2 Initialize a Git Repository

- Navigate to your project directory:

- cd /path/to/your/project

- Initialize a new Git repository:

- git init

1.3 Add Configuration Files

- Add your PLC and HMI configuration files to the repository:

- git add plc_config.txt hmi_config.xml

1.4 Commit Changes

- Commit the changes to the repository with a descriptive message:

- git commit -m "Initial commit for PLC and HMI configs"

Task 2: Record Changes in a Configuration Management Database

2.1 Install SQLite

- Install SQLite on your system.

- Windows: Download SQLite-tools and unzip the package. Add the directory to your PATH.

- Linux:

- sudo apt-get install sqlite3

2.2 Create a Configuration Management Database

- Create and connect to a new SQLite database:

- sqlite3 config_management.db

2.3 Create a Table to Store Configuration Information

- Within the SQLite interactive shell, execute the following command to create a table:

- CREATE TABLE ConfigChanges (

- ID INTEGER PRIMARY KEY AUTOINCREMENT,

- FileName TEXT NOT NULL,

- DateModified TEXT NOT NULL,

- ChangeDescription TEXT NOT NULL

- );

2.4 Inserting Change Records into the Database

- Insert records detailing configuration changes:

- INSERT INTO ConfigChanges (FileName, DateModified, ChangeDescription)

- VALUES ('plc_config.txt', '2023-10-20', 'Updated ladder logic for safety interlock');

Task 3: Conduct Periodic Reviews to Ensure Configurations Match the Recorded Versions

3.1 Schedule Regular Reviews

- Use a task scheduler (e.g., cron jobs for Linux, Task Scheduler for Windows) to automate the review process.

3.2 Automated Script for Review

- Create a script to compare the current configuration files with those recorded in the database:

  - Here's a Bash script example:

- #!/bin/bash

- # Review Configurations

- current_hash=$(git rev-parse HEAD)

- # Fetch the hash from the database or file (this would require additional setup)

- stored_hash="stored_hash_value"

- if [ "$current_hash" == "$stored_hash" ]; then

- echo "Configurations are consistent with the recorded versions."

- else

- echo "Discrepancy found! Review needed."

fi

Lab 40: ICS/SCADA Compliance & Standards Overview

Objectives

- Understand the ISA/IEC 62443 series and its relevance to Industrial Control Systems (ICS) security.

- Explore the NERC CIP standards and their application in the power sector.

- Identify how compliance frameworks can influence security policies and practices.

Prerequisites

- Basic knowledge of ICS/SCADA systems.

- Familiarity with cybersecurity principles.

- Access to a text editor and a command-line interface for running scripts.

Tasks Overview

Task 1: Discuss ISA/IEC 62443 Series for ICS Security

Subtask 1.1: Introduction to ISA/IEC 62443

- The ISA/IEC 62443 series provides a comprehensive framework for securing ICS from cyber threats.

- Key objectives include ensuring the availability, integrity, and confidentiality of industrial networks and systems.

Subtask 1.2: Key Components of ISA/IEC 62443

- Security Lifecycle: Emphasizes continuous assessment and improvement.

- Security Levels: Defines varying degrees of security controls.

- Zones and Conduits: Establishes different security zones to control data flow.

Subtask 1.3: Case Study - Application in Industrial Sectors

- Examine a real-world application where ISA/IEC 62443 was successfully implemented to enhance ICS security in a manufacturing plant.

Task 2: Briefly Cover NERC CIP for Power Sector

Subtask 2.1: Overview of NERC CIP Standards

- NERC CIP (Critical Infrastructure Protection) standards aim to protect the North American bulk power system.

- Focus on securing IT and ICS to prevent and mitigate impacts of cyber events.

Subtask 2.2: Breakdown of NERC CIP Requirements

- CIP-002 to CIP-009: Cover areas such as asset identification, security management controls, and incident reporting.

- Discuss the significance of each requirement and its role in securing power systems.

Subtask 2.3: Example - NERC CIP Implementation

- Explore a case where a utility company adopted NERC CIP standards to boost cybersecurity resilience.

Task 3: Identify How Compliance Frameworks Impact Security Policies

Subtask 3.1: Compliance Frameworks and Security Policies

- Framework Influence: Compliance frameworks guide the creation and refinement of security policies.

- Highlight key considerations when aligning policies with standards like ISA/IEC 62443 and NERC CIP.

Subtask 3.2: Code Snippet - Security Policy Template

- Provide a basic example of a security policy aligned with compliance standards:

# Sample Security Policy Template

policy:

name: ICS_Security_Policy

version: 1.0

description: Policy for managing ICS security according to ISA/IEC 62443 and NERC CIP.

controls:

- id: SL1

description: Minimum security level established for network zones.

- id: IM1

description: Incident management procedures for cyber threats.

enforcement:

- reviewPeriod: Quarterly

- revisionCondition: Significant changes in ICS infrastructure